SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA0703

Name: T. Venkata Reddy
Reg no: 192412567

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Annexure A

ANALYTICAL PROBLEM SOLVING

- A university network is assigned the IP address block 192.168.20.0/24 for its internal communication. The network administrator has decided to divide this network into subnets for three different labs using fixed subnet masks. Lab A is assigned a /26 subnet, Lab B is assigned a /27 subnet, and Lab C is assigned a /28 subnet. Based on the given subnet masks, apply subnetting concepts to determine the network address of each lab. Further, identify the valid range of host IP addresses and the broadcast address for each subnet. Finally, calculate the total number of usable host IP addresses available in each lab’s subnet.

Given:

Network: 192.168.20.0/24

Lab A → /26

Lab B → /27

Lab C → /28

Lab A (/26)

Subnet Mask: 255.255.255.192

Network Address: 192.168.20.0

Host Range: 192.168.20.1 – 192.168.20.62

Broadcast Address: 192.168.20.63

Total IPs = 64

Usable Hosts = 62

Lab B (/27)

Subnet Mask: 255.255.255.224

Network Address: 192.168.20.64

Host Range: 192.168.20.65 – 192.168.20.94

Broadcast Address: 192.168.20.95

Total IPs = 32

Usable Hosts = 30

Lab C (/28)

Subnet Mask: 255.255.255.240

Network Address: 192.168.20.96

Host Range: 192.168.20.97 – 192.168.20.110

Broadcast Address: 192.168.20.111

Total IPs = 16

Usable Hosts = 14

2. A network administrator has configured two systems with the following details: System 1 has IP address 192.168.1.130 with subnet mask 255.255.255.192, and System 2 has IP address 192.168.1.190 with subnet mask 255.255.255.192. Using the given configuration, apply subnetting techniques to determine the subnet range, network address, and broadcast address for each system. Identify the valid host IP range for both subnets based on the given subnet mask. Calculate whether each IP address falls within its respective subnet range. Also, determine the number of usable host addresses available in each subnet.

Given that

Subnet Mask:

- 255.255.255.192 = /26

- Block Size = 64

Subnets:

- 192.168.1.0 – 63

- 192.168.1.64 – 127

- 192.168.1.128 – 191

- 192.168.1.192 – 255

System 1

IP Address: 192.168.1.130

- Network Address: 192.168.1.128

- Host Range: 192.168.1.129 – 192.168.1.190

- Broadcast: 192.168.1.191

- IP 192.168.1.130 is Valid

- Usable Hosts = 62

System 2

IP Address: 192.168.1.190

- Network Address: 192.168.1.128

- Host Range: 192.168.1.129 – 192.168.1.190

- Broadcast: 192.168.1.191

- IP 192.168.1.190 is Valid

- Usable Hosts = 62

3. An organization has been allocated the IP block 172.16.0.0/16 for its internal network. The network administrator has already decided the subnet masks for each department as follows: HR → /25, Sales → /26, IT → /27, and Admin → /28. Using the given subnet masks, apply subnetting techniques to assign suitable subnet addresses for each department. Determine the network address, valid host IP range, and broadcast address for each subnet. Based on the allocation, calculate the number of usable host addresses in each department. Finally, identify the remaining unused IP address range within the given network block.

Given:

- Network: 172.16.0.0/16

HR (/25)

- Network: 172.16.0.0

- Host Range: 172.16.0.1 – 172.16.0.126

- Broadcast: 172.16.0.127

- Usable Hosts: 126

Sales (/26)

- Network: 172.16.0.128

- Host Range: 172.16.0.129 – 172.16.0.190

- Broadcast: 172.16.0.191

- Usable Hosts: 62

IT (/27)

- Network: 172.16.0.192

- Host Range: 172.16.0.193 – 172.16.0.222

- Broadcast: 172.16.0.223

- Usable Hosts: 30

Admin (/28)

- Network: 172.16.0.224

- Host Range: 172.16.0.225 – 172.16.0.238

- Broadcast: 172.16.0.239

- Usable Hosts: 14

Remaining Unused IP Range

Unused addresses:

172.16.0.240 – 172.16.255.255

4. A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a /27 subnet mask for its internal departments. Using the given subnet mask, apply subnetting techniques to calculate the total number of subnets created and the number of usable hosts per subnet. Determine the subnet to which the IP address 172.16.10.78 belongs. Identify the network address and broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

Given:

- Network: 172.16.10.0/24

- Subnet Mask: /27

(a) Total Number of Subnets

Borrowed Bits = 27 − 24 = 3

Number of Subnets =

2³ = 8

(b) Usable Hosts Per Subnet

Host Bits = 5

Hosts =

2⁵ − 2 = 30

(c) Find Subnet of 172.16.10.78

Block Size = 32

Subnets:

- 0–31

- 32–63

- 64–95

- 96–127

- 128–159

- 160–191

- 192–223

- 224–255

Therefore,

- Network Address = 172.16.10.64

- Host Range = 172.16.10.65 – 172.16.10.94

- Broadcast = 172.16.10.95

(d) Does 172.16.10.95 belong to same subnet?

Yes.

172.16.10.95 is the broadcast address of the subnet 172.16.10.64/27, so it belongs to that subnet but cannot be assigned to a host.

5. A network uses the Internet Protocol version 6 address 2001:0db8:0000:0000:0000:ff00:0042:8329. Apply IPv6 addressing rules to compress the given address into its shortest form and then expand the compressed address back to its full notation. Using a prefix length of /64, determine the network prefix of the given address. Also, calculate the total number of possible host addresses available within this network based on the given prefix length.
Given Address:

2001:0db8:0000:0000:0000:ff00:0042:8329

(a) Compressed Form

Remove leading zeros and replace consecutive zero blocks with "::"

2001:db8::ff00:42:8329

(b) Expanded Form

2001:0db8:0000:0000:0000:ff00:0042:8329

(c) Network Prefix (/64)

Network Prefix:

2001:0db8:0000:0000::/64

(d) Number of Host Addresses

Host Bits =

128 − 64 = 64

Total Host Addresses:

2⁶⁴ = 18,446,744,073,709,551,616

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |