SIMATS ENGINEERING

ASSESSMENT TOOL 3

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA0703

Name: T. Venkata Reddy

Reg no: 192412567

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Software:Cisco Packet Tracer,Wireshark

Annexure A

Debugging & Optimisation Task

1. A network administrator has configured a workstation with the following settings:

- IP Address: 192.168.10.65

- Subnet Mask: 255.255.255.192 (/26)

- Default Gateway: 192.168.10.1

However, the workstation is unable to communicate with other systems configured in the subnet 192.168.10.0/26.

Task

Debug the network configuration by:

- Identifying the subnet to which the configured IP address belongs.

- Determining the network address, broadcast address, and valid host range.

- Verifying whether the configured IP address and gateway are valid.

- Identifying the configuration errors.

- Optimizing the configuration by assigning an appropriate IP address and default gateway.

- Calculating the total number of usable host addresses after correction.

Given:

- IP Address: 192.168.10.65

- Subnet Mask: 255.255.255.192 (/26)

- Default Gateway: 192.168.10.1

Step 1: Identify the Subnet

A /26 subnet has a block size of 64.

Subnets:

- 192.168.10.0 – 192.168.10.63

- 192.168.10.64 – 192.168.10.127

- 192.168.10.128 – 192.168.10.191

- 192.168.10.192 – 192.168.10.255

The IP 192.168.10.65 belongs to the subnet 192.168.10.64/26.

Step 2: Network Details

- Network Address: 192.168.10.64

- Broadcast Address: 192.168.10.127

- Valid Host Range: 192.168.10.65 – 192.168.10.126

Step 3: Verify IP Address and Gateway

- IP Address 192.168.10.65 is valid.

- Default Gateway 192.168.10.1 belongs to subnet 192.168.10.0/26, not 192.168.10.64/26.

Step 4: Configuration Error

The default gateway is in a different subnet, so the workstation cannot communicate correctly.

Step 5: Optimized Configuration

- IP Address: 192.168.10.65

- Default Gateway: 192.168.10.126 (or any valid router IP within 192.168.10.65–126)

Step 6: Usable Host Addresses

Formula:
2^(32−26) − 2 = 64 − 2 = 62

Total usable hosts = 62

2. Two devices are configured with the following IPv6 addresses:

- Device A: 2001:db8::ff00:42:8329/64

- Device B: 2001:db8:0:0:1::1/64

Although both devices are connected to the same network, communication fails.

### Task

Debug the IPv6 configuration by:

- Expanding both IPv6 addresses into their complete notation.

- Determining the network prefix of each device.

- Verifying whether both devices belong to the same subnet.

- Identifying the addressing or prefix mismatch.

- Optimizing the IPv6 configuration by assigning appropriate addresses.

- Calculating the number of available host addresses within the corrected subnet.

Given:

- Device A: 2001:db8::ff00:42:8329/64

- Device B: 2001:db8:0:0:1::1/64

Step 1: Expanded Addresses

Device A

2001:0db8:0000:0000:0000:ff00:0042:8329

Device B

2001:0db8:0000:0000:0001:0000:0000:0001

Step 2: Network Prefix

- Device A Prefix: 2001:db8:0:0::/64

- Device B Prefix: 2001:db8:0:0::/64

Step 3: Same Subnet?

No.

With a /64 prefix, the first four blocks are the network and the last four blocks are the interface ID.

- Device A interface ID: 0000:ff00:0042:8329

- Device B interface ID: 0001:0000:0000:0001

The 0001 appears in the fifth block, which is part of the interface ID. Both devices still share the same /64 network prefix 2001:db8:0:0::/64. If communication fails, it is likely due to another configuration issue (such as incorrect routing, Neighbor Discovery, firewall settings, or the intended address assignment).

Step 4: Addressing Issue

If the intention was for both devices to be simple hosts on the same LAN, use interface IDs that clearly belong to the same prefix.

Step 5: Optimized Configuration

- Device A: 2001:db8:0:0::10/64

- Device B: 2001:db8:0:0::20/64

Step 6: Available Host Addresses

A /64 network provides:

2⁶⁴ = 18,446,744,073,709,551,616 interface addresses

3. An organization has allocated the network 192.168.1.0/24 into several departments using the subnet masks /26, /27, and /28. Some departments report insufficient IP addresses, while others have a large number of unused addresses.

Task

Analyze and optimize the subnet allocation by:

- Identifying the network address, broadcast address, and host range for each subnet.

- Calculating the number of usable and unused IP addresses.

- Identifying inefficient subnet allocations.

- Debugging the addressing plan by locating subnet allocation mistakes.

- Recommending an optimized subnetting scheme using VLSM to improve address utilization while supporting current departmental requirements.

Subnet 1 (/26)

- Network Address: 192.168.1.0

- Broadcast Address: 192.168.1.63

- Host Range: 192.168.1.1 – 192.168.1.62

- Total Addresses: 64

- Usable Hosts: 62

Subnet 2 (/27)

- Network Address: 192.168.1.64

- Broadcast Address: 192.168.1.95

- Host Range: 192.168.1.65 – 192.168.1.94

- Total Addresses: 32

- Usable Hosts: 30

Subnet 3 (/28)

- Network Address: 192.168.1.96

- Broadcast Address: 192.168.1.111

- Host Range: 192.168.1.97 – 192.168.1.110

- Total Addresses: 16

- Usable Hosts: 14

Unused Addresses

- /26 → 2 reserved addresses (network and broadcast)

- /27 → 2 reserved addresses

- /28 → 2 reserved addresses

If departments use significantly fewer hosts than the subnet supports, many addresses remain unused, resulting in inefficient allocation.

Inefficient Allocation

- Small departments assigned a /26 waste many addresses.

- Large departments assigned a /28 may not have enough addresses.

Debugging

Possible mistakes:

- Incorrect subnet boundaries.

- Overlapping subnets.

- Wrong subnet mask.

- Duplicate IP addresses.

- Gateway outside the subnet.

Optimized VLSM Scheme

Allocate subnets according to department size:

- Largest department → /26 (62 hosts)

- Medium department → /27 (30 hosts)

- Small department → /28 (14 hosts)

This minimizes address wastage while meeting host requirements.

4. Two devices are configured with the following IPv6 addresses:

- Device A: 2001:db8::ff00:42:8329/64

- Device B: 2001:db8:0:0:1::1/64

Although both devices are connected to the same network, communication fails.

Task

Debug the IPv6 configuration by:

- Expanding both IPv6 addresses into their complete notation.

- Determining the network prefix of each device.

- Verifying whether both devices belong to the same subnet.

- Identifying the addressing or prefix mismatch.

- Optimizing the IPv6 configuration by assigning appropriate addresses.

- Calculating the number of available host addresses within the corrected subnet.

Expanded Addresses

Device A:

2001:0db8:0000:0000:0000:ff00:0042:8329

Device B:

2001:0db8:0000:0000:0001:0000:0000:0001

Network Prefix

Both use:

2001:db8:0:0::/64

Verification

Both devices have the same /64 network prefix. If communication fails, verify interface configuration, Neighbor Discovery, router advertisements, and firewall settings.

Optimized Addresses

- Device A: 2001:db8:0:0::10/64

- Device B: 2001:db8:0:0::20/64

Available Hosts

2⁶⁴ = 18,446,744,073,709,551,616 interface addresses

5. A router contains the following routing table.

| Destination Network | Next Hop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| Default Route | 10.0.0.1 |

Users report that packets destined for 192.168.2.100 are not reaching the intended network.

Task

Debug the routing configuration by:

- Identifying the matching routing entry using the longest-prefix match.

- Determining the next-hop router selected for packet forwarding.

- Verifying whether the destination IP belongs to the intended subnet.

- Identifying any routing configuration errors.

- Optimizing the routing table to improve packet forwarding efficiency.

- Identifying the route that will be used if no matching destination exists.

Step 1: Longest-Prefix Match

Destination IP:
192.168.2.100

It matches:

192.168.2.0/24

Step 2: Selected Next Hop

10.0.0.3

Step 3: Verify Destination Subnet

192.168.2.100 belongs to:

- Network: 192.168.2.0/24

- Host Range: 192.168.2.1 – 192.168.2.254

Therefore, the destination belongs to the intended subnet.

Step 4: Routing Errors

Possible causes:

- Incorrect next-hop IP.

- Next-hop router is unreachable.

- Missing return route.

- Interface is down.

- Incorrect subnet mask.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Identification | 20% | Accurately identifies all errors and root causes with clear understanding | Identifies most errors with minor gaps | Identifies some errors but misses key issues | Unable to identify errors correctly |
| Debugging | 25% | Applies systematic debugging techniques effectively to resolve issues | Uses appropriate debugging methods with minor errors | Limited debugging approach; requires guidance | Unable to debug or resolve issues |
| Optimization | 20% | Optimizes code by significantly improving time complexity using efficient algorithms | Improves time complexity with minor gaps in efficiency | Limited improvement in time complexity | No improvement; inefficient approach retained |
| Analysis | 20% | Demonstrates strong logical reasoning and clear justification of solutions | Shows reasonable analysis with some justification | Limited analysis; weak reasoning | No analytical reasoning demonstrated |
| Code Quality | 15% | Produces clean, well-structured code with proper comments and documentation | Code is mostly clear with minor documentation gaps | Code lacks structure and proper documentation | Poorly written code with no documentation |