SIMATS ENGINEERING

ASSESSMENT TOOL 1

Experiments

COURSE NAME: Computer Networks			COURSE CODE:	CSA0703		Name: T. Venkata Reddy                                   Reg no:192412567

COURSE OUTCOME COVERED

CO3: Analyze the performance of core network protocols such as TCP, UDP, HTTP, and DNS under varying conditions

Assessment Tool Weightage: 40%

SOFTWARE: CISCO PACKET TRACER, WIRESHARK

EXPERIMENTS

QUESTIONS:4						                Total Marks:40

- Design a simple LAN consisting of two PCs connected through a switch using Cisco Packet Tracer. Configure appropriate IP addresses for both hosts and verify connectivity using the ping command. Simultaneously capture the packet exchange using Wireshark. Identify the Ethernet frame fields such as Source MAC Address, Destination MAC Address, EtherType, and Frame Check Sequence (FCS). Explain how the Data Link layer uses MAC addressing to ensure successful frame delivery within the local network.

Network Topology

PC0 -------- Switch -------- PC1

IP Configuration

- PC0

  - IP Address: 192.168.1.10

  - Subnet Mask: 255.255.255.0

- PC1

  - IP Address: 192.168.1.20

  - Subnet Mask: 255.255.255.0

Steps

- Open Cisco Packet Tracer.

- Place two PCs and one switch.

- Connect each PC to the switch using Copper Straight-Through cables.

- Configure the IP addresses on both PCs.

- Open Command Prompt on PC0.

- Run:

ping 192.168.1.20

- Open Wireshark and capture packets on the Ethernet interface during the ping.

Ethernet Frame Fields

- Source MAC Address: MAC address of the sender (PC0).

- Destination MAC Address: MAC address of the receiver (PC1).

- EtherType: 0x0800 (IPv4).

- Frame Check Sequence (FCS): Used to detect transmission errors.

- Create a network topology consisting of two LANs connected through a router in Cisco Packet Tracer. Configure IPv4 addressing, subnet masks, and default gateways for all devices, and verify end-to-end communication using the ping command. Capture the ICMP packets using Wireshark and analyze the IPv4 header fields, including Source IP Address, Destination IP Address, Time-To-Live (TTL), Protocol Number, and Header Length. Explain the role of the Network layer in forwarding packets between different networks.

Network Topology

PC0 ---- Switch ---- Router ---- Switch ---- PC1

IP Configuration

LAN 1

- PC0: 192.168.1.10/24

- Router G0/0: 192.168.1.1/24

LAN 2

- PC1: 192.168.2.10/24

- Router G0/1: 192.168.2.1/24

Default Gateways

- PC0: 192.168.1.1

- PC1: 192.168.2.1

Steps

- Build the topology.

- Configure IP addresses and subnet masks.

- Configure router interfaces.

- Test connectivity:

ping 192.168.2.10

- Capture ICMP packets in Wireshark.

IPv4 Header Fields

- Source IP Address: 192.168.1.10

- Destination IP Address: 192.168.2.10

- TTL (Time-To-Live): 128 (or operating system default)

- Protocol Number: 1 (ICMP)

- Header Length: 20 bytes

- Construct a local area network with multiple hosts connected through a switch in Cisco Packet Tracer. Clear the ARP cache on one of the hosts and initiate communication with another host using the ping command. Capture the communication in Wireshark and identify the ARP Request and ARP Reply packets. Analyze the sender and target MAC/IP addresses and explain how the Address Resolution Protocol resolves logical IP addresses into physical MAC addresses before data transmission.

Network Topology

PC0 ----- Switch ----- PC1 ----- PC2

IP Configuration

- PC0: 192.168.1.10

- PC1: 192.168.1.20

- PC2: 192.168.1.30

Steps

- Configure all hosts.

- Clear the ARP cache:

arp -d

- Ping another host:

ping 192.168.1.20

- Capture packets using Wireshark.

ARP Packets

ARP Request

- Sender IP: 192.168.1.10

- Sender MAC: PC0 MAC

- Target IP: 192.168.1.20

- Target MAC: 00:00:00:00:00:00 (unknown)

ARP Reply

- Sender IP: 192.168.1.20

- Sender MAC: PC1 MAC

- Target IP: 192.168.1.10

- Target MAC: PC0 MAC

- Develop a simple network consisting of two PCs connected through a router in Cisco Packet Tracer. Configure the required IP addresses and test connectivity using the ping command. Capture the ICMP traffic using Wireshark and identify the Echo Request and Echo Reply messages. Examine important ICMP fields such as Type, Code, Identifier, and Sequence Number, and calculate the Round Trip Time (RTT). Discuss how ICMP assists in network troubleshooting and connectivity verification.

Network Topology

PC0 -------- Router -------- PC1

IP Configuration

- PC0: 192.168.1.10/24

- Router G0/0: 192.168.1.1

- Router G0/1: 192.168.2.1

- PC1: 192.168.2.10/24

Steps

- Configure all IP addresses.

- Configure default gateways.

- Ping:

ping 192.168.2.10

- Capture packets using Wireshark.

ICMP Fields

- Echo Request

  - Type: 8

  - Code: 0

  - Identifier: Unique value

  - Sequence Number: Starts at 1

- Echo Reply

  - Type: 0

  - Code: 0

  - Identifier: Same as request

  - Sequence Number: Same as request

Round Trip Time (RTT)

Example:

- Request sent: 10.200 ms

- Reply received: 10.206 ms

RTT = 10.206 − 10.200 = 6 ms

- Create a client-server network in Cisco Packet Tracer by configuring a web server and a client PC. Enable the HTTP service on the server and access the hosted webpage from the client using a web browser. Capture the communication in Wireshark and identify the TCP three-way handshake, followed by the HTTP GET request and HTTP response messages. Analyze the application-layer communication and explain how TCP provides reliable delivery for HTTP traffic through connection establishment, acknowledgments, and ordered data transfer.

Network Topology

Client PC -------- Switch -------- Web Server

IP Configuration

Client

- IP Address: 192.168.1.10

- Subnet Mask: 255.255.255.0

Web Server

- IP Address: 192.168.1.20

- Subnet Mask: 255.255.255.0

Steps

- Add one PC, one switch, and one server.

- Connect all devices using Copper Straight-Through cables.

- Configure IP addresses.

- On the server, enable the HTTP service.

- Open the web browser on the client.

- Enter:

http://192.168.1.20

- Capture traffic using Wireshark.

TCP Three-Way Handshake

- SYN – Client requests a connection.

- SYN-ACK – Server acknowledges and accepts.

- ACK – Client confirms the connection.

HTTP Communication

- HTTP GET Request: Client requests the webpage.

- HTTP Response: Server sends the webpage (status code 200 OK).

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25 | Demonstrates thorough understanding and effectively integrates multiple concepts/technologies | Good understanding with proper integration of most concepts | Basic understanding; limited or partial integration | Poor understanding; unable to integrate concepts |
| Experimental Design | 30 | Well-planned, systematic implementation with optimal solution | Correct implementation with minor issues | Basic implementation with errors or inefficiencies | Incorrect or incomplete implementation |
| Use of Tools | 20 | Effective and advanced use of tools/technologies with proper configuration | Appropriate use of tools with correct execution | Limited or inconsistent use of tools | Incorrect or minimal use of tools |
| Analysis of Results | 15 | Insightful analysis with accurate interpretation and validation of results | Good analysis with reasonable interpretation | Basic analysis with limited insights | Poor or incorrect analysis of results |
| Documentation | 10 | Clear, well-structured documentation with diagrams, code, and results | Organized documentation with necessary details | Basic documentation with missing details | Poor or incomplete documentation |