| Q.No | Question | Marks | CO | BLT |
| --- | --- | --- | --- | --- |
| 1 | A 3-input majority gate returns a true output if at least two of the inputs are true. A minority gate is its complement. Design a 3-input CMOS minority gate using a single stage of logic. a) Sketch a transistor-level schematic b) sketch a stick diagram c) Estimate the area from the stick diagram. | 10 | CO 1 | BLT 3 |
| 2 | Sketch a 3-input NAND with transistor widths chosen to achieve effective rise and fall resistances equal to a unit inverter (R). | 10 | CO 1 | BLT 4 |
| 3 | Sketch a 2-input NAND gate with transistor widths chosen to achieve effective rise and fall resistance equal to that of a unit inverter (R). Compute the raising and falling propagation delays (in terms of R and C) of NAND gate driving h identical gates using the Elmore delay model. If C=2fF/µm, R=2.5kΩ in a 180nm process. What is the delay of a fan-out of 3 input NAND gate. | 10 | CO 1 | BLT 3 |
| 4 | Use the method of logical effort to minimize the delay of branching path A → Y. a. Determine path logical effort path G, electrical effort H, path branching effort B, and path parasitic delay P. b. Compute best stage effort. C. Compute minimum path delay. | 10 | CO 1 | BLT 4 |
| 5 | Consider the circuit below. a. Determine path logical effort path G, electrical effort H, path branching effort B, and path parasitic delay P. b. Compute best stage effort. c. Compute minimum path delay. | 10 | CO 1 | BLT 4 |

|  | 1590 5 15 90 |  |  |  |
| --- | --- | --- | --- | --- |
| 6 | Consider the two designs of a 2-input AND gate shown in Figure. Give an intuitive argument about which will be faster. Back up your argument with a calculation of the path effort, delay, and input capacitances x and y to achieve this delay. | 10 | CO 1 | BLT 5 |
| 7 | A 3-input majority gate returns a true output if at least two of the inputs are true. A minority gate is its complement. Design a 3-input CMOS minority gate using a single stage of logic. | 10 | CO 1 | BLT 3 |
| 8 | Fig shows two designs for an 8-input domino AND gate using footed dynamic gates. One uses four stages of logic with static CMOS inverters. The other uses only two stages by employing a HI-skew NOR gate. For what range of path electrical efforts is the 2-stage design faster? | 10 | CO 1 | BLT 5 |
| 9 | Sketch a 4-input NAND gate with transistor widths chosen to achieve equal rise and fall resistance as a unit inverter. Show why the logical effort is 6/3. | 10 | CO 1 | BLT 4 |
| 10 | Figure models a gate driving wires to two destinations. The gate is represented as a voltage source with effective resistance R1. The two receivers are located at nodes 3 and 4. The wire to node 3 is long enough that it is represented with a pair of π-segments, while the wire to node 4 is represented with a single segment. Find the Elmore delay from input x to each receiver. (BTL4) | 10 | CO 1 | BLT 4 |